| Building & Location | Description |
| --- | --- |

| Crouch Hill Farmhouse, Holwell Drove (listed as Church Hill sic), G2 1304895 | Possibly C17 origin – stone elevations under a hipped, thatched roof, central brick stack, 2 storey. |
| --- | --- |
| Westrow Cottage East, Holwell Drove, G2 1324136 | 2 storey house, part early, part mid C17. Coursed rubble walls with tiled gable-ended roofs and end stacks. Modern 2 storey extension to east in similar style. |
| Westrow Cottage West, Holwell Drov , G2 1152182 | 2 storey house, formerly 2 cottages, dating from 1707. Coursed rubble with C20 tiled roof, gabled and hipped. Brick stacks to right ends of original cottages. |
| Holwell Manor, Holwell Drove, G2* 1118845 | C16 country house mostly rebuilt 1889. Coursed, squared rubble, stone tiled, gable ended roof and various stacks. 2 storey plus attic. Stone mullioned windows. |
| Buckshaw House, Buckshaw (Stock Hill Lane), G2* 1152103 | House, mid C18 with extensive C19 alterations and additions. Imposing façade with central pavilion. Rendered and ashlar walls with stone quoins, hipped slate roof with brick stacks. 2 storey and attic. |
| Buckshaw Farmhouse, G2 1118842 | Farmhouse, early C19. Rendered walls, hipped slate roof with 2 stage brick stacks springing from eaves. Symmetrical. 2 storeys. |
| Lower Buckshaw Farmhouse, G2 1324134 | Farmhouse, late C17 or early C18. Coursed rubble walls, asbestos slate roof with end brick stacks. Main range near symmetrical.3 storeys. To the right is a 2 storey extension. |
| The Rectory, The Borough, G2* 1304902 | Early C18 house, rendered walls, stone quoins, hipped tiled roof with stone verges and brick stacks. Moulded plaster coves beneath eaves. 2 storey. Good quality mid C18 joinery. Sash windows with glazing bars. |
| Holwell Farmhouse, The Borough, G2 1118844 | Late C17, course rubble walls, thatched half-hipped roof. Brick stacks to right gable and part way in from left. Adjacent cowshed now converted to domestic use. |
| Borough Cottage, The Borough, G2 1324135 | Mid C18. Rubble walls, tiled, gable ended roof, end brick stacks. 2 storey. |
| Naish Farm , Stony Lane, G2* 1118847 | C15, alterations and additions C16, C18 and CF19. Rubble walls with thatched roof, gable ended to right and hipped to left. Brick stack right and part way along eaves to left. C15 north door of elm planks and other C15 features |
| Elm Tree Farmhouse and outbuildings to East, Packers Hill , G2 1324137 | C18 probably. Brick with ashlar dressings, gable ended tiled roof, stone copings, brick end stacks. 2 storey with attic. Attached – 2 ranges of brick outbuildings, probably early C19. |
| Strawberry Cottage, Packers Hill , G2 1118846 | 2 cottages, now one house. Early C18 and C19. Thatched, half-hipped roof with brick stack. 2 storey. |
| Middle Piccadilly Farmhouse, Middle Piccadilly, G2 1152205 | Early C19 rendered colour-washed walls, thatched roof, brick end stacks. 2 storey. Central gabled. |
| Woodbridge Farmhouse, G2 1118848 | Rear early C18, front later C18. Rear wing of brick with plat-band and cornice. Front rubble, thatched with gable ends, end brick stacks, 2 storey. Also pair of farmbuildings, 20m to south of farmhouse. G2 1152259 late C18/C19. |
| Church of St Laurence, The Borough, G1 1152109 | Late C15 Parish Church, restored 1885. Coursed limestone rubble (Marnhull quarries ?), ashlar dressings, gable ended stone roofs / stone copings. C15 barrel vault nave roof. West tower with plinth and embattled parapet. C15 carved angel capitals bearing scrolls, late C17 pulpit. |
| Table tomb 9m East of the Chancel to the Church, G2 1264161 | Dated 1607, stone with chamfered flat top. To Robert Bridges. |
| Table tomb 4m South of South aisle to Church, G2 1118843 | C17 stone with moulded flat top. No visible inscription. |
| Bridge (part in Bishops Caundle) 60m North of St Laurence Church, G2 1118883 | Probably of Medieval origin, with early C19 northern arch. Massive central pier of rubble with flat top and pointed cutwaters. Former 1950’s iron girders to south span now removed, and whole top replaced with timbered walkway. |
| Cornford Bridge, Caundle Brook, Cornford Hill , G2 * 1324112 | Multi-span stone road bridge c1480 with repairs and restorations of C18, 1994 and early C21. Rare – significant example of a Medieval span bridge of which there are less than 200 known surviving examples in England. “An ongoing testament to medieval span bridge building in England “. Particularly fine detail and stonework, and the design has a strong ecclesiastical influence. |
| Rowden Mill Bridge, Rowden Mill Lane (pt Holwell, pt Stourton Caundle), G2 1152245 | Footbridge, over Caundle Brook, mid C19. Coursed rubble with 2 round arches. Pointed cut water with flat top. Ridged ashlar coping. Coursed rubble ramps on both sides. |
| Pillar Box at Barnes Cross, G2* 1237251 | 1853 cast iron polygonal with vertical slit. Manufactured by John Butt & Co, Gloucester. Believed to be the oldest in public service in the UK. Possibly relocated from Sherborne. |